Welder, Combination

APPRENTICESHIP APPROACH

Competency

O*NET-SOC CODE

51-4121.00

RAPIDS CODE

0622CB

| Work Process Schedule | SKILLS |  |
| --- | --- | --- |
| Maintain safety. Operate safety equipment and use safe work habits. Measure dimensions of completed products or workpieces to verify conformance to specifications. Examine workpieces for defects and measure workpieces with straightedges or templates to ensure conformance with specifications. Check grooves, angles, or gap allowances, using micrometers, calipers, and precision measuring instruments. Operate welding equipment. Weld components in flat, vertical, or overhead positions. Recognize, set up, and operate hand and power tools common to the welding trade, such as shielded metal arc and gas metal arc welding equipment. Weld separately or in combination, using aluminum, stainless steel, cast iron, and other alloys. Select and install torches, torch tips, filler rods, and flux, according to welding chart specifications or types and thicknesses of metals. Ignite torches or start power supplies and strike arcs by touching electrodes to metals being welded, completing electrical circuits. Guide and direct flames or electrodes on or across workpieces to straighten, bend, melt, or build up metal. Notify others of equipment repair or maintenance needs. Detect faulty operation of equipment or defective materials and notify supervisors. Watch operating equipment to detect malfunctions. Select production equipment according to product specifications. Determine required equipment and welding methods, applying knowledge of metallurgy, geometry, and welding techniques. Clean workpieces or finished products. Prepare all material surfaces to be welded, ensuring that there is no loose or thick scale, slag, rust, moisture, grease, or other foreign matter. Mark products, workpieces, or equipment with identifying information. Align parts or workpieces to ensure proper assembly. Align and clamp workpieces together, using rules, squares, or hand tools, or position items in fixtures, jigs, or vises. Melt metal, plastic, or other materials to prepare for production. Melt and apply solder along adjoining edges of workpieces to solder joints, using soldering irons, gas torches, or electric-ultrasonic equipment. Solder parts or workpieces. Adjust equipment controls to regulate gas flow. Connect and turn regulator valves to activate and adjust gas flow and pressure so that desired flames are obtained. Monitor equipment operation to ensure that products are not flawed. Monitor the fitting, burning, and welding processes to avoid overheating of parts or warping, shrinking, distortion, or expansion of material. Mount materials or workpieces onto production equipment. Position and secure workpieces, using hoists, cranes, wire, and banding machines or hand tools. Operate grinding equipment. Chip or grind off excess weld, slag, or spatter, using hand scrapers or power chippers, portable grinders, or arc-cutting equipment. Clean or degrease parts, using wire brushes, portable grinders, or chemical baths. Grind, cut, buff, or bend edges of workpieces to be joined to ensure snug fit, using power grinders and hand tools. Reshape metal workpieces to established specifications. Repair products by dismantling, straightening, reshaping, and reassembling parts, using cutting torches, straightening presses, and hand tools. Cut industrial materials in preparation for fabrication or processing. Ignite fuel to activate heating equipment. Trim excess material from workpieces. Operate firefighting equipment. Use fire suppression methods in industrial emergencies. Design templates or patterns. Develop templates and models for welding projects, using mathematical calculations based on blueprint information. Disassemble equipment for maintenance or repair. Heat material or workpieces to prepare for or complete production. Preheat workpieces prior to welding or bending, using torches or heating furnaces. Clean production equipment. Assemble temporary equipment or structures. Set up and use ladders and scaffolding as necessary to complete work. Shape metal workpieces with hammers or other small hand tools. Review blueprints or other instructions to determine operational methods or sequences. Analyze engineering drawings, blueprints, specifications, sketches, work orders, and material safety data sheets to plan layout, assembly, and operations. | 52 |  |
| Totals | SKILLS 52 | OJT HRS 0 |