Electronic Security Technician

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

49-2098.00

RAPIDS CODE

2007

| Work Process Schedule |  | OJT HRS |
| --- | --- | --- |
| Install electrical components, equipment, or systems. Install, maintain, or repair security systems, or related equipment, following blueprints of electrical layouts and building plans. Mount and fasten control panels, door and window contacts, sensors, or video cameras, and attach electrical and telephone wiring to connect components. |  | 300 |
| Explain use of products or services. Demonstrate systems for customers and explain details, such as the causes and consequences of security alarms. Review of user documentation manuals &amp; instructions Identify training objectives Confirm actual user and their requirements Procure or develop training &amp; user aids, manuals, etc Demonstrate system functions by guiding users through the system Observe user using the system Communicate results of training back to all relevant parties |  | 250 |
| Position equipment using hand tools, power tools, or heavy equipment. Mount and fasten control panels, door and window contacts, sensors, or video cameras, and attach electrical and telephone wiring to connect components. |  | 100 |
| Repair electrical components. Install, maintain, or repair security systems, or related equipment, following blueprints of electrical layouts and building plans. Remote location components Central/main location |  | 250 |
| Repair electrical circuits or wiring. Test and repair circuits and sensors, following wiring and system specifications. Power up/Power Down Operate and test functions Evaluate Performance |  | 300 |
| Test electrical circuits or components for proper functioning. Test and repair circuits and sensors, following wiring and system specifications. Identify problems, errors, discrepancies Diagnose causes of problems Take remedial action Document Actions |  | 250 |
| Inspect equipment to locate or identify electrical problems. Examine systems to locate problems, such as loose connections or broken insulation. |  | 100 |
| Lay cables to connect equipment. Feed cables through access holes, roof spaces, or cavity walls to reach fixture outlets, positioning and terminating cables, wires, or strapping. |  | 250 |
| Inspect safety equipment to ensure proper functioning. Test backup batteries, keypad programming, sirens, or other security features to ensure proper functioning or to diagnose malfunctions. |  | 150 |
| Drill holes in parts, equipment, or materials. Drill holes for wiring in wall studs, joists, ceilings, or floors. Pre-termination functions Connect Passive devices Splicing Intermediate Terminations |  | 750 |
| Determine types of equipment, tools, or materials needed for jobs. Inspect installation sites and study work orders, building plans, and installation manuals to determine materials requirements and installation procedures. |  | 100 |
| Plan work procedures. Inspect installation sites and study work orders, building plans, and installation manuals to determine materials requirements and installation procedures. Perform site survey Develop overall job plan Organize technical work plan Complete pre-assemblies &amp; fabrication of sub systems Gather Inventory/parts |  | 200 |
| Document operational activities. Prepare documents, such as invoices or warranties. |  | 100 |
| Confer with customers or users to assess problems. Consult with clients to assess risks and to determine security requirements. |  | 150 |
| Run wiring to connect equipment. Mount raceways and conduits and fasten wires to wood framing, using staplers. Use Documentation to lay out components Secure work area &amp; Safety area security Rough in device component locations Install cable support structure Label tag wire/cable per documentation |  | 300 |
| Adjust equipment to ensure optimal performance. Adjust sensitivity of units, based on room structures and manufacturers' recommendations, using programming keypads. |  | 150 |
| Estimate costs for labor or materials. Provide customers with cost estimates for equipment installation. |  | 150 |
| Maintain knowledge of current developments in area of expertise. Keep informed of new products and developments. |  | 100 |
| Order materials, supplies, or equipment. Order replacement parts. |  | 50 |
| Totals |  | OJT HRS 4K |

Electronic Security Technician

APPRENTICESHIP APPROACH

Time

O*NET-SOC CODE

49-2098.00

RAPIDS CODE

2007

| Related Instruction Online | RSI HRS |
| --- | --- |
| Certified Alarm Technician Level 1 (CAT 1) | 30 |
| OSHA 30 | 25 |
| Understanding Electronic Security Systems | 35 |
| Life Safety Code | 15 |
| Wireless Systems 101, 102 & 103 | 10 |
| Fundamentals of Networking | 8 |
| Low Voltage Cabling | 20 |
| Fire Alarm Installation Methods | 20 |
| Testing Security Systems | 5 |
| Introduction to Construction Drawings | 20 |
| International Building Code | 18 |
| Video Systems Technology | 20 |
| Advanced Intrusion Systems | 20 |
| Networking 101, 102, 103, 104, 105, 106 | 10 |
| Electronic Access Control | 20 |
| Troubleshooting Intrusion Alarm Devices | 5 |
| Job Safety and False Alarm Reduction | 5 |
| Troubleshooting, Service and Maintenance | 5 |
| Troubleshooting Fire Devices | 5 |
| Power and Electricity with National Electrical Code | 10 |
| Totals | RSI HRS 310 |